Kapiti Coast Erosion Hazard Assessment 2012 Update : Shand, R.D., August 2012

A Critical Assessment

Summary of contents and notes 26 September : J.N. Ashby

- GENERAL OVERVIEW

  - INTRODUCTION

Introduce empirical tenets that appear to be broken and define documentation required by Guidelines including IPCC models. Accept sea-level rise and emphasise not skeptics. Coastal erosion is a non-exact science and difficult to model. Accretionary coastline. Coastal erosion is a non-exact hazard unlike earthquake / liquefaction, tsunami inundation hazards.

Define documentation required by Guidelines.

Terminology utilise Gibb (2002).

Erosion vs inundation : disregard of the contour levels

Methodology empirically flawed with production of parallel equidistant 50 and 100 year lines : non-real

Does dune restoration become part of managed or unmanaged (semi-natural)

  - ANALYSIS OF SHAND OVERVIEW

General analysis of the Shand overview which seems to contain a number of irregularities.

General comments that need to be contested ;

Terms of reference (p7) : “scientifically robust and defendable”

Disclaimer

Indication that the methodology may be over-estimating (p4)

Indication that there was inconsistency in results (p4) : Otaki River

“conversion process undertaken by KCDC staff” (p4) : when was this done

“including adequate public consultation” (p4) : not done

“CSL were instructed to … take into account … littoral drift” (p2) : not done

Draft June 2011 … focus group required addition … who was focus group

- GUIDANCE MANUAL

2.1 MINISTRY FOR THE ENVIRONMENT 2008 GUIDELINES

Analysis of the use of the Guidance Manual as there appears to have been misinterpretation.

Incorrect interpretation of Table 2.3

3.1.3 (p18) conveniently ended up with a 50yr value of 0.3 (same as 2008 report) no need to recalculate.

Wilkinson email to Aitchison and reply 25 Sep

2.2  IPCC (2007) DETERMINATIONS

Should use this data to be scientifically acceptable : could easily get aligned with sea level rise skeptics.

Indications from Naish that IPCC (2013) will show an upward revision.

New IPCC data does not apply under regulations at this stage.

Argument, especially on this coast, that average IPCC data should be used (not pessimistic)

- OPEN COAST EROSION HAZARD ANALYSIS

In this section do the detailed analysis essentially following Shands subsections

  - METHODS

3.1.1  LONGER TERM SHORELINE CHANGE

3.1.1.1  GEOLOGIC INDICATORS

Kapiti Coast borders a coastal plain constructed over approximately the last 7,000 years (Gibb 2002).

Coastal plain constructed by a supply of sediment predominated over sea-level changes (Gibb 1994).

Gibb (2002) section 3.4 Waikanae coastline : 7000 to 1800yr BP : 0.42m/yr

Then 1800 to 1100 : 0.43m/yr and 1100 to present : 0.64m/yr (increase due to deforestation Gibb 1978).

Radiocarbon dates from Otiran Periglacial (Fleming 1970).

Marine cliff (te Punga 1962) : Waikanae – Waterfall Stream – Te Horo : 3000 to 8000 years

Hawke and McConchie (2006) : 0.48m/yr

3.1.1.2  PRE-HISTORIC INDICATORS

Adkin (1948) Waitarere Beach : 2 ft per year for the last 600 to 800 years

3.1.1.3  HISTORIC INDICATORS

The wreck of the Fusilier in Santoft Forest : Cowie (1963) : 3 ft per year for the last 80 years

The wreck of the Hyderabad Waitarere Beach Gibb (2002) last century Waikanae and Paraparaumu Beach 0.16 to 1.08m/yr.

Waters Edge accretion in recent  years (Gibb 2002).

Waikanae Estuary profiles 1.23m/yr (Gibb 2002).

Jessup data : September 2012 email : Bryant Farm Pekapeka

Maori gates at Te Atiawa Park originally on banks of Waikanae River.

Aerial photographic analysis (first in 1942 shows Olive Terrace as part of estuary).

River outlet at Manly Gardens 1942.

Absence of inland dune forms north of Manly St Dairy indicates river presence.

Dawe (2012) progradation at Waimeha : 0.4 to 0.8m/yr since 1942.

Dawe (2012) since dune restoration southern Waimeha up to 30m in 10 years.

3.1.1.4  CUSPATE FORELAND

The dominant feature of this coastal plain is the cuspate foreland in the shadow of Kapiti Island.

Giant offshore breakwater, reducing wave energy from the prevailing northwest.

Wave generated longshore currents lose velocity and deposit sand to form the cusp (Gibb 1978).

The Kapiti Coast cuspate foreland is the largest of its kind in New Zealand (Gibb 2002).

Guidance Manual indicates a cuspate foreland has LOW sensitivity to climate change (Jay et al 2003).

Cuspate foreland migration (Gibb 2002).

Tentative deposition of 1 million cu m per year on subtidal banner bank.

3.1.1.5  DISCUSSION

Setting LT to zero will be main discussion point

3.1.2  SHORT TERM EROSION

Frequency of 15 to 20 years for erosion episodes (Gibb 2002).

El-Nino Southern Oscillation (ENSO) : 2-7 year cycle.

Interdecadal Pacific Oscillation (IPO) ; 20-30 year cycle.

3.1.3  SHORELINE RETREAT ASSOCIATED WITH SEA LEVEL RISE

3.1.3.1  BRUUN RULE

Shands own comments

Andrew, Cooper, and Pilkey (2004) : Katherine

De Lange original blog and subsequent comments.

Bring in heavyweight analysis here.

3.1.3.2  LONGSHORE DRIFT

Gibb (1978) mineralogy of the sand.

Detrital mineral percentages indicate a dominance of the greywacke source.

Sediment from rivers discharging to the coast between Egmont and Waikanae.

Some supply from eroding seacliffs between Cape Egmont and Wanganui.

Pebble imbrication at Rangitikei River mouth (te Punga 1953).

The fine mudstones from the Wanganui Basin tend to discharge as suspension mud.

Sand deposition in the near-shore zone (out to about 20m water depth).

Transported both alongshore and offshore by the action of waves and currents.

Holland and Holland (1985) indicated a northeasterly longshore drift direction since disproven.

Hypersthene andesite pebbles on Paraparaumu Beach derived from Ruapehu.

Volcanic derived heavy minerals with diagnostic tracer minerals from the Egmont Volcanic Zone.

Greywacke gravels at Otaki River mouth indicate net southerly drift (Gibb 1980).

The Waikanae River mouth historically has always migrated south.

Subtidal banner bank is actively growing southwestward from the apex of the cuspate foreland.

Reversals in drift direction will occur but waves energy diminished by Wellington west coast and Kapiti. Coast is fully exposed to relatively larger waves from the north-northwest.

Net southerly wave generated longshore drift at the Waikanae River mouth (Gibb 2002).

Analysis of river mouth deflections from south of Wanganui (GoogleEarth) gives southward deflection.

Shand (p20) sediment dynamic study in progress could affect long-term : but LT ignored when positive

3.1.3.3.  SEDIMENT SUPPLY FROM WAIKANAE RIVER

River history : 1890 blockage of northern branch : 1920s Waimeha Stream outlet.

Firth gravel extraction at bridge.

Greater Wellington consent for Otaihanga flood mitigation.

Sediment transport mechanism (Gibb 2002).

Data from Waikanae River only best estimates (2200 cu m per year).

Natural and man cutting through at estuary.

Data from North Beach group (2002 2003) to be included (also in longshore drift section).

Sedimentation plates currently in Waikanae River : data would be useful

3.1.3.4  DISCUSSION

Heavyweights come in here too

3.1.4  DUNE STABILITY

Need help here but generally not that significant

3.1.5  UNCERTAINTY AND OTHER FACTORS

3.1.5.1  TECTONIC UPLIFT

Fleming (1972) analysis.

Get more recent data on glacio-isostatic rebound

3.1.5.2  INCREASED SEDIMENT SUPPLY FROM PRECIPITATION

3.1.5.3  INCREASED SEDIMENT SUPPLY FROM WANGANUI COASTAL CLIFF EROSION

3.1.5.4  DISCUSSION

- CONCLUSIONS

- REFERENCES

CASE STUDY : WAIKANAE ESTUARY

Need 2008 report to assess raw data : requested copy from Guerin 25 Sep

OTHER CASE STUDIES COULD BE ADDED INTO HERE