Report OF WILLEM PIETER de LANGE

Qualifications and Experience

- My full name is Willem Pieter de Lange. I hold the degrees of BSc (1981), MSc(Hons) First Class Honours (1983), and DPhil (1989) from the University of Waikato in the fields of Computer Sciences and Earth and Ocean Sciences.

- I am employed as Senior Lecturer at the University of Waikato within the Department of Earth and Ocean Sciences, Faculty of Science and Engineering, a position that I have held since 1984.

- During my scientific career I have published some 179 scholarly items, including: 93 peer-reviewed published chapters and scientific journal papers, mainly in the international scientific journals, presented some 9 statements of expert evidence for Hearing / Planning Tribunal / Environment and High Court cases, and published an additional 38 consulting reports. I have supervised and co-supervised 92 MSc/MSc(Tech), 2 MPhil and 18 DPhil/PhD students to completion.

- I am also a member of the Coasts, Oceans, Ports and Rivers Institute (COPRI) of the American Society of Civil Engineers (ASCE), the Coastal Education and Research Foundation (CERF), The Oceanographic Society (TOS), and the NZ Coastal Society and NZ Society for Earthquake Engineering technical groups of the Institute of Professional Engineers New Zealand (IPENZ).

- My training was in Earth and Ocean Sciences, particularly in sedimentology and coastal processes, and Computer Science, particularly numerical modelling, data analysis and visualisation.

- My Masters and Doctoral research were based at Tauranga Harbour. Firstly, my MSc thesis examined tsunami hazard in the Bay of Plenty, and involved numerical modelling of tsunami generation, propagation and impacts along the coastline and within Tauranga Harbour (de Lange, 1983). My DPhil thesis dealt specifically with the behaviour of dredge spoil used to renourish Pilot Bay Beach, Mt Maunganui, but also compared the performance of the two numerical models used for the Tauranga Harbour Study (2DD and DHI System 21) against additional validation data obtained for my research (de Lange, 1988; de Lange & Healy, 1991). My DPhil also assessed the contribution of wave-induced sediment transport, particularly over the flood tidal delta (Centre Bank) and intertidal flats, to the total sediment transport within the Harbour.

- Since 1984, mostly at the University of Waikato, I have undertaken research into fundamental coastal processes and management pertaining to New Zealand estuaries and the coast. During that time my research has concentrated on coastal hazards, including tsunamis, storm surges, meteo-tsunamis, waterspouts, sea level variations, climate fluctuations, and coastal erosion processes and mitigation.

- This research has including investigations into the methodologies used to determine coastal hazard zones for a range of different types of coast from sandy beaches to cliffed coasts. I have undertaken determinations of coastal hazard zones.

Scope of Report.

- I have considered three reports produced by Coastal Systems Ltd for the Kapiti District Council concerning coastal hazard zonation of the Kapiti District coastline. In particular,in this report two issues are addressed;

- Were their deficiencies  in the CSL Reports that would have been uncovered if adequately peer reviewed?

- Did they include appropriate consideration of risk to properties within the proposed hazard zones?

Independent peer review

- Peer review is an important process in providing a degree of confidence in the outcome of reports and publications of scientific investigations. There are different intensities of review, but generally a review will examine whether or not the methodology followed conforms to accepted best practice, and whether or not the results appear reasonable. It is not usual to replicate the analyses undertaken, but this may be necessary depending on the purpose to which the results will be put.

- For the determination of coastal hazard zones within New Zealand at the time that Coastal Systems Ltd undertook their analyses (published in 2008 and 2012), there was no formal definition of acceptable best practice. However, there were guidelines available, including:

- Published peer reviewed papers and books dealing with coastal hazard zones, such as Kay and Alder (1999) or Healy and Dean (2000);

- Local Government Guidance Notes published by the Ministry for the Environment in 2004, and in revised form in 2008 (Ministry for the Environment, 2008); and

- Technical manuals produced by Regional Councils for the management of coastal hazards such as by the ARC Coastal Resources Group and Beca Carter Hollings and Ferner Ltd (2000)

- Ramsay et al. (2012) produced a supplementary document to the Local Government Guidance Notes, which is the first formal proposal for a standard best practice for the determination of coastal hazard zones in New Zealand.

- However, in my opinion it will be difficult to define a standard methodology that covers all coastal situations in New Zealand. .

There is a wide range of coastal types in New Zealand. The methodologies presented by Ramsay et al. (2012) are intended primarily for sandy beach systems, and are not necessarily appropriate for other types of coastline, such as mixed sand-gravel beaches, gravel or boulder beaches, chenier and muddy coasts, and hard and soft rock cliffs.Kapiti Coast has a mixture of coastlines.

It is recognised in the literature that the response of the coastline is a function of the net sediment budget, and that a sediment surplus or deficit can be more important than any other factor. This is referred to in Ramsay et al. (2012) and Ministry for the Environment (2008), but not explicitly included in the proposed methodology. It has been demonstrated for regions with a long-term history of surplus sediment, the shoreline is likely to continue to accrete. Often these regions have developed salients, such as cuspate forelands and tombolos, which are resistant to climate variations.

The science underpinning the various aspects of coastal hazards, and hence the associated coastal hazard zones, is evolving and not certain. For example, there are no predictions of future climate. Instead there is a range of projections based on various scenarios. It is clear that, at most, one of these scenarios is likely to be correct, but there is no method for assessing which one it will be.

Therefore, it is imprudent to base a methodology on an expected outcome. Also, while it is recognised that the net sediment budget is important, the sediment budgets for the New Zealand coast are poorly constrained. [ explain costrained}Further, the impact of future climate on coastal sediment budgets is even less well understood.

- Partly for the reasons listed above, it is common practice to recommend that the coastal hazard zones be periodically reviewed, usually after 10-15 years. This recommendation was made for the hazard zones proposed by Coastal Systems Ltd.

- Without a nationally accepted formal definition of the appropriate methodology for the determination of coastal hazard zones, the peer review is a critical step in assessing if the results are meaningful and suitable for the intended final use. In my opinion, as the consequences of an incorrect assessment increase, then the intensity of the peer review should also increase and eventually include replication of key analyses.

Matters a Peer Reviewer would be expected to comment on re Coastal Systems Ltd reports

- As far as I can determine, the 3 reports produced by Coastal Systems Ltd were only subjected to a superficial internal review by Dr Shepherd (who also works for Coastal Systems Ltd). However, there were more substantial reviews of earlier work commissioned by the Kapiti District Council that led to the 3 reports. These reviews did raise issues about possible methodologies and suitable data that I assume contributed to the selection of the final methodology by Dr Shand.

- I have examined the methodology used by Coastal Systems Ltd. I have determined that there are a disturbing number of ways in which the methodology deviates from what I would consider to be standard practice.  I discuss two of these issues.

- Accretion

- Firstly, Dr Shand makes an assumption that any coastal regions that display a long-term trend of accretion should be treated as having a net trend of zero. Having assigned a fixed, and hence known value with no error, to the long-term rate, he then applies an empirically derived uncertainty term for the long-term rate of -3.7 m per 50 years. This has the effect of converting a long-term accreting trend to an artificial erosional trend that then biases the coastal hazard zone towards higher erosion. In contrast, for areas with existing seawalls, Dr Shand sets the net trend and uncertainty to zero, except at site CS12.50 where the seawall was buried by accretion.

- At section 3.1.1 of his 2012 report Dr Shand provides his rationale for this approach as … ‘a precautionary measure used by the industry in recognition of the uncertainty inherent in predicting sustained seaward shoreline migration over prolonged periods of time where the underlying process is not well understood.”

- This assumption of ignoring accretion is unusual, and in my opinion unwarranted and not standard practice. Dr Jeremy Gibb previously questioned this assumption as a peer reviewer of the earlier KCDC Open Coast Erosion Hazard Report (2007).

- Dr Shand provides three different explanations for this assumption:

“In an effort to simplify the computation method - thereby facilitating hazard update by future council staff, ... [a]ll positive (acretionary) long-term rates of change have been set to 0” (Page 23 Summary of Peer Reviewer comments on the KCDC Open Coast Erosion Hazard Report, 2007).

- In my opinion, the accretion of the shoreline provides a reduction of the coastal hazard. Ignoring the accretion inflates the apparent hazard, and has no basis in reality.A peer review would have picked this up. I would expect a review to ask for evidence to justify the assertion  that this is a common approach.

Secondly he said “Where positive rates occur, LT [long-term rate] is set at zero, this being a precautionary measure used by the industry in recognition of the uncertainly inherent in predicting sustained seaward shoreline migration over prolonged periods of time where the underlying process is not well understood” (Pages 16-17, 2012 Update report).

Thirdly he said “Of particular note is that for all areas subject to a positive (seaward) shoreline trend, the rate was set to zero. This approach is common when assessing hazards for accreting coasts as it removes the assumption of continued accretion, provides [sic] an increased safety margin” (Page 24, 2008 Part 1 report).

- . There is sufficient evidence to show that: (a) the Kapiti cuspate foreland is a Holocene feature that developed over the last 7000 years, and in part it is established on older material that has been accreting for at least 11,000 years (viz. Hawke and McConchie, 2005);  (b) there have been significant climatic and sea level variations over that time (viz. Clement et al., 2010; Lewis et al., in press); (c) there is no evidence of prehistoric severe widespread coastal erosional events in the development of the foreland (viz. Hawke and McConchie, 2006) or the Manawatu coast supplying sediment to Kapiti (viz. Clement et al., 2010), although there is evidence for three periods of dune disruption with the last two linked to human activity (viz. Hawke and McConchie, 2006). Therefore, in my opinion it would be reasonable to conclude that, with no evidence to the contrary, the observed accretion is likely to continue, albeit with possible variations in the rate of accretion.

- . Although the rate of accretion cannot be reliably predicted, on the basis of the available evidence it is likely to be greater than zero over 50-100 years. Therefore, it is not acceptable to assume a zero accretion rate that in combination with an uncertainty of -3.7 m per 50 years produces an apparent long-term erosion trend.

- . Further, I consider it contradictory to claim that accretion (and other process, such as tectonics) should be ignored because they are unpredictable, but erosion, which is also unpredictable, must be. In my opinion, all factors that can be identified should be included in the analysis.

- . Ramsay et al. (2012) indicate that a minimum of 50 years is required to define the long-term trend for a shoreline. Implicit in this assumption, is an expectation that changes in the long-term trend will take longer than 50 years to become apparent. The assumption made by Dr Shand is that in response to sea level rise or some other unknown factor, the shoreline will switch from accretion to erosion at the date of calculation of the coastal hazard zone.A peer reviewer to make an appropriate comment.

- . In my opinion, the null hypothesis should be that the observed long-term trends will continue into the foreseeable future. It is therefore necessary to demonstrate with an appropriate level of confidence that there will be a change in the future. Dr Shand has not attempted to do that. I would have expected a peer reviewer to make an appropriate comment.

- Administrative convenience

27. Dr Shand further justified zero rating accretion on the basis of administrative convenience saying,

“In an effort to simplify the computation method - thereby facilitating hazard update by future council staff, ... [a]ll positive (acretionary) long-term rates of change have been set to 0” (Page 23 Summary of Peer Reviewer comments on the KCDC Open Coast Erosion Hazard Report, 2007).

- 28. In my opinion, convenience for non-experts is a  non scientific justification for an assumption that materially affects the outcome, and I would have expected a peer reviewer to make an appropriate comment.

- Acceleration of sea level rise

- 29. Another issue relates to the estimation of the effect of any future acceleration in the rate of sea level rise above that which occurred during the historical record. Dr Shand has taken a published methodology by Komar et al. (1999) that was developed to assess storm erosion, and modified it to assess the impact of sea level rise. The orginal method did not include a sea level rise term at all. It is based on the extreme water level relative to the elevation of the dune toe, and the beach lowering during the storm. It is not intended to predict the effects of long-term sea level rise, but is intended to assess short-term shoreline fluctuations (cut and fill) during storms as demonstrated by Ramsay et al. (2012). Figure 29 in Ramsay et al. (2012) is a diagram I prepared for my undergraduate students to illustrate the methodology and includes the key equation (Figure 1).

- Dr Shand modified the Komar et al.  (1999) equation to produce Equations 3 and D2 in the 2008 reports. He doesn’t explain his derivation, but examination of the original and the derived forms show that he replaced the numerator term, which related to the super-elevation of mean sea level during a storm and beach surface lowering, with sea level rise (Equation 3 in the 2012 report).

- Therefore, instead of using the equation as defined,  he used the ratio of the sea level rise to the nearshore slope, which is equivalent to the equation originally proposed by Bruun (1962). The original Bruun equation has been modified over time (viz. SCOR Working Group 89, 1991), mostly to use different measures for the nearshore slope, ranging from the gradients of: (a) the inner continental shelf (Bruun, 1962); the active beach defined by the extent of sediment exchange between the beach and the shelf (Bruun, 1988); the active surf zone (Dean et al., 1987); the offshore bar (Dubois, 1977); and the beach from the dune crest to outer surf zone margin (Weggel, 1979). There are also variations to account for mixtures of grain sizes, effects of cohesion, and landward transport of sediment (viz. Bruun, 1983; Rosati et al., in press).

- The large number of modifications that were made to relate Bruun rule predictions with observed changes, led the SCOR Working Group (1991) to recommend that “The Bruun rule be used only for order-of-magnitude estimates of potential shoreline recession rates. Large error bars should be included with any calculated estimates as a reminder of the approximate nature of the analysis procedure. If possible, the full budget of sediments should be developed for the study site so that the factors can be included in predictions of the shoreline changes”.

- . Dr Shand in the 2008 open coast report states

- “However, the model has come under increasing criticism in recent years regarding its underlying assumptions. While conditions on parts of the east coast of New Zealand are more suited to meeting these assumptions, its application to the New Zealand west coast is more questionable because of the existence of reefs and longshore sediment transfer (littoral drift), neither of which are allowed for in the model’s assumptions” (Page 32, 2008 Part 1 report).

- . Dr Shand presented a more detailed discussion of the Bruun method in Appendix D of the same report. This was based on Bruun (1983), which discusses the limitations of the approach, and presented a method to estimate the slope of the beach (Equation D1), so that (L/[B+d]) is equal to 1/tanβ. This version of the Bruun rule was proposed by Weggel (1979), and was intended to approximate the beach face (tanβ in the Oregon beach method illustrated in Figure 1). Hence, Equations D1 and D2 (and 3) are functionally identical, and Dr Shand applied a method he also identified as being inappropriate. I would have expected a peer reviewer to make an appropriate comment.

- Dr Shand  considered that the Komar method (and presumably his modification of it) is better than the Bruun rule because it uses the intertidal section of the beach profile (the beach face), even though it is intended for a different purpose. However, the Bruun rule as expressed in Equation D1 of his report was also uses the slope of the beach face (Weggel, 1979) by taking B as the height of the beach berm and d as the depth at the edge of the active surf zone. His modified equation is therefore the same as the Bruun Rule he extracted from Bruun (1983) and not appropriate for the determination of the shoreline response at Kapiti, as explained by Dr Shand.

- . As explained by the SCOR Working Group 89 (1991)

- “In that the derivation of the Bruun Rule is independent of the nearshore profile, the resulting equations consititute little more than a landward migration of the nearshore zone up the regional slope, and accordingingly can be expected to be correct within one’s ability to choose the appropriate θ [corresponds to β in Dr Shand’s report and this affidavit] value”.

- . The Working Group also emphasise the need to compare the estimates from the Bruun rule with observations. Dr Shand has not done this. It is evident that the Bruun rule will fail to correctly predict the historical shoreline response for sites that have accreted, and those with seawalls. I would have expected a peer reviewer to have made an appropriate comment.

- . Therefore, in my opinion the internal review undertaken by Coastal Systems Ltd was not sufficient to provide confidence that the coastal hazard zones have been correctly determined in accordance with best practice. I also note, that any deficiencies in the methodology will be amplified over the time of the prediction. Thus the, reliability of the 100 year predictions both are necessarily less than for the 50 year predictions.

Risk assessment  Coastal Systems Ltd reports

- . Strictly hazard refers to the potential for harm to people and their property, and for the coastal hazards of coastal erosion and inundation the focus tends to be on property (Lundgren, 1986). So following this definition, a coastal zone would be a region where a specific type of event has a potential to cause harm, which is the approach taken by Ramsay et al. (2012). Therefore, the coastal hazard zone would be only an identification of potential (possible) hazards with no consideration of their probability of occurrence.

- . In practice, hazard zones are generally associated with risk, which may be defined as a function of the probability of occurrence for a given hazard, and the consequences arising from a hazard event. It has been common practice with coastal hazard zones to associate the hazard with some level of probability, such as the 1% Annual Exceedence Probability (Kay and Alder, 1999; Ramsay et al., 2012). Further, it is common to distinguish between “Greenfield” sites with minimal consequences and developed sites with moderate to high consequences.

- . Without some associated probability, coastal hazard lines are of little practical value because it is possible to conceive of unlikely hazards that can affect large regions. For example, the entire coastal plain within the Kapiti District has a potential liquefaction hazard. However, liquefaction hazard depends on the probability of an earthquake (of the order of 0.005-0.0025% per annum) and the susceptibility of any particular location, and so for many areas may be considered to be an acceptable level of risk. Hence, defining the whole plain as a liquefiable zone is not useful.

- . Therefore, coastal hazard zones are often evaluated in terms of risk for specified hazards, and referred to as development setback lines (Healy and Dean, 2000). This requires that an assessment of the level of risk, and some decision on the level of acceptable risk should be included in the quantification of coastal hazard zones.

- . The New Zealand Coastal Policy Statement 2010 states that it is necessary to “Identify areas in the coastal environment that are potentially affected by coastal hazards (including tsunami), giving priority to the identification of areas at high risk of being affected”.

- . In my opinion, this implies that there should be an identification and prioritisation of risk associated with coastal hazards. In that regard, I do not agree with Ms Thompson at paragraph 46 where she suggests (from a planning perspective) that “coastal erosion tends not to be expressed probabilistically.”

- No quantification of risk.

- . The Coastal Systems Ltd reports do not quantify risk. Instead they address uncertainties, and avoid any meaningful quantification of the probabilities associated with the results. For example, Dr Shand states in relation to the long-term trend, “While some hazard assessments incorporate such statistics, this was not done in the present assessment as the weighting procedure, together with the variance reduction measures of setting positive rates to zero and the selection of the maximum longshore rate, were found to be adequate” (Page 26, 2008 Part 1 report).

- . Further, the uncertainties that were estimated, are combined together into a single parameter that is added to the estimated erosion. This represents a one-sided, and hence, biased uncertainty. In a risk based hazard assessment uncertainties are normally assumed to be two-sided (can increase or decrease the hazard). In relation to the long-term erosion rate, when Dr Shand fixes the accretion rate at zero (a value with no uncertainty) and applies an estimated one-sided uncertainty, he creates a long-term erosion trend. This approach creates an erosion hazard where none existed in the historical record, and no measure of risk is included. I would have expected a peer reviewer to make an appropriate comment

- .  It is not possible to predict shoreline accretion into the future, and the same is also true for erosion. Therefore, as recommended by SCOR Working Group 89 (1991) for predicting shoreline response to sea level rise, there should be large error bars to indicate the uncertainty of the estimates. It is not reasonable to claim it is a potential outcome

- . In 2012 a revised assessment of the coastal hazard zones was published to address the NZ Coastal Policy Statement 2010 directives. There was a change in terminology, so that the 2012 report produced Coastal Hazard Prediction Lines. This implies that there should be some quantification of the confidence limits for the predictions used to locate the lines. These confidence limits are not presented in the report, and are not represented by the uncertainty calculations presented in the reports.

- . The 2012 report also considers specific time periods for the predictions (50 and 100 years), and the effects of different management approaches for coastal areas currently protected by structures. The inclusion of the requirement to consider time periods implies a requirement to present a risk analysis that considers the appropriate factors such as magnitude, frequency and impact. This is not provided in the 2012 report and I would have expected a peer reviewer to make an appropriate comment.

- . Dr Shand said “An erosion hazard assessment such as this is a detailed scientific study which predicts potential hazard (magnitude and probability) over different prediction periods. By contrast, erosion management is a process whereby the science-based erosion predictions are transformed into hazard management zones for inclusion within the District Plan. This conversion process is presently being undertaken by the KCDC planning staff who also factor in a range of other matters including the requirements of statues, regulations and planning provisions, recognizing that different types of development carry different risk levels and the need to provide for reasonable use of existing property, and recognizing the future needs of the community and including adequate public consultation” (Page 4, 2012 update report).

- . This statement recognises that the potential hazard should be quantified in terms of the magnitude and probability.

- . The approach of combining terms in this way is statistically incorrect and overly conservative, and in my opinion this applies to the entire methodology used to create the coastal hazard prediction lines.

- . The other implication of the statement made on page 4 of the 2012 update report, is that there was an expectation that a subsequent risk assessment would be made by the Kapiti Coast District Council. As far as I can determine, this was not done. Therefore, there is no valid risk assessment associated with the coastal hazard prediction lines adopted by the Kapiti Coast District Council.

- Precautionary approach

- . The so called precautionary as required by the New Zealand Coastal Policy Statement 2010. (Policy 3(1) was invoked by Dr Shand.

- Policy 3(1) states:

- : ‘Adopt a precautionary approach towards proposed activities whose effects on the coastal environment are uncertain, unknown, or little understood, but potentially significantly adverse.”

- .  As noted above, in relation to coastal hazard assessments there is in my opinion a clear requirement to assess the risk. The precautionary approach referred to in the Coastal Policy Statement  relates to the impacts of new coastal activities which in my opinion is not applicable to existing infrastructure or coastal processes.

- . None of the reports quantify the hazard, except to assert that the values chosen are “precautionary”. I am not aware of any specific probability assigned to this term. Summing a series of factors with precautionary probabilities is assuming that all will occur simultaneously. As noted by Dr Shand in relation to his uncertainty calculations “… to assume that they are likely to occur at the same time is statistically incorrect and gives an overly conservative combined uncertainty value” (Page 38, 2008 part 1).

- Managed adaptive approach

- . The UK Environment Agency took 6 years to develop the TE2100 plan for managing flooding and coastal hazards and other issues along the Thames Estuary and River (Environment Agency, 2012). The affected area covered 350 km2, with 1.25 million residents and £200 billion property value in 2012. The Environment Agency evaluated a range of different approaches to dealing with future risks particularly climate change. This assessment found that the more extreme climate-related projections were extremely unlikely to occur, and hence the cost of adopting them for a one-off precautionary measure was not justified. Instead, the plan involves a managed adaptive approach where measures are introduced over time to manage the risk. The interventions can be modified as and when the risks are better quantified and understood (Figure 2).

- . In opinion, if it is not possible to quantify the risk associated with the coastal hazard zones proposed, then a managed adaptive approach is a more appropriate way to deal with coastal hazards than single precautionary intervention at the start.

- Reliability of the predictions

- . I understand that the time the Council made its decision it was working on the basis that the 50 year and 100 year lines reflected “likely” future beach lines. Subsequently it is has been accepted that this is not the case and the information on LIMs simply refers to “projected” future shorelines”. The information does not indicate that these are “worst case scenarios”, though this is how Dr Shand describes them.

- . In my opinion the lines are cannot be used to reliably predict worst case scenarios out to 100 years. Furthermore, for historically accreting parts of the coastline,  both the 50 and 100 year predictions are likely to be exaggerated and accordingly are not even worst case scenarios. A proper independent peer review would have debated and resolve issues around the methodology and the commented on the reliability of the predictions.

- . As noted earlier, there was an expectation by Dr Shand, that a subsequent risk assessment would be made by the Kapiti Coast District Council. As far as I can determine, this was not done. Therefore, there is no valid risk assessment associated with the coastal hazard prediction lines adopted by the Kapiti Coast District Council.

References

ARC Coastal Resources Group and Beca Carter Hollings and Ferner Ltd, 2000. Coastal hazard strategy:  coastal erosion management manual. Auckland Regional Council Technical Publication 130, Auckland.

Bruun, P., 1962. Sea level as a cause of shore erosion. Journal of Waterways and Harbours Division, ASCE, 88: 117-130.

Bruun, P., 1983. Review of conditions for uses of the Bruun Rule of erosion. Coastal Engineering, 7: 77-89.

Bruun, P., 1988, The Bruun rule of erosion by sea-level rise:  a discussion on large-scale two- and three-dimensional usages: Journal of Coastal Research, 4: 627-648.

Clement, A.J.H., Sloss, C.R., and Fuller, I.C. 2010. Late Quaternary Geomorphology of the Manawatu Coastal Plain, North Island, New Zealand. Quaternary Land-Ocean Interactions: Driving Mechanisms and Coastal Responses, special issue of Quaternary International 221 (1–2): 36–45. doi:10.1016/j.quaint.2009.07.005.

Dean, R.G., Dalrymple, R.A., Fairbridge, R.W., Leatherman, S.P., Nummedal, D., O'Brien, M.P., Pilkey, O., Sturges III, W., and Wiegel, R.L., 1987, Responding to changes in sea level: Washington, D.C., Committee on Engineering Implications of Changes in Relative Mean Sea Level, Marine Board, Commission on Engineering and Technical Systems, National Research Council, National Academy Press, 148 p.

Dubois, R.N., 1977, Predicting beach erosion as a function of rising water level. Journal of Geology, 85: 470-476.

Environment Agency, 2012. TE2100 Plan: Managing flood risk through London and the Thames Estuary, Thames Estuary 2100: London, Environment Agency, p. 226.

Healy, T.R. and Dean, R.G., 2000. Methodology for delineation of coastal hazard zones and development setback for open duned coasts. In: J.B. Herbich (Editor), Handbook of coastal engineering, pp. 19.1-19.30.

Hawke, R.M., and McConchie, J.A.., 2005. The Source, Age, and Stabilisation of the Koputaroa Dunes, Otaki‐Te Horo, New Zealand. New Zealand Journal of Geology and Geophysics 48(3): 517–522.

Hawke, R. M., and McConchie, J.A., 2006. Dune Phases in the Otaki-Te Horo Area (New Zealand): a Geomorphic History.”Earth Surface Processes and Landforms 31(6): 633–645.

Kay, R. and Alder, J., 1999. Coastal planning and management. E & FN Spon, London, 375 p.

Komar, P.D., McDougal, W.G., Marra, J.J. and Ruggiero, P., 1999. Rational analysis of setback distances: applications to the Oregon Coast. Shore and Beach, 67(1): 41-49.

Lewis, S.E., Sloss, C.R., Murray-Wallace, C.V., Woodroffe, C.D., and Smithers, S.G., Post-glacial sea-level changes around the Australian margin: a review: Quaternary Science Reviews. doi:j.quascirev.2012.09.006

Lundgren, L., 1986. Environmental Geology. Prentice-Hall, Englewood Cliffs, New Jersey, 576 pp.

Ministry for the Environment (2004). Coastal Hazards and Climate Change. A Guidance Manual for Local Government in New Zealand. Prepared for the Ministry for the Environment by NIWA, Beca Consultants Ltd, DTec Consultants Ltd, and Tonkin and Taylor Ltd . Wellington. xi +145 p.

Ministry for the Environment (2008). Coastal Hazards and Climate Change. A Guidance Manual for Local Government in New Zealand. 2nd edition Revised by Ramsay, D., and Bell, R. (NIWA) Prepared for the Ministry for the Environment. Wellington. viii +127 p.

Ramsay, D.L., Gibberd, B., Dahm, J., Bell, R.G. (2012) Defining coastal hazard zones and setback lines. A guide to good practice. National Institute of Water & Atmospheric Research Ltd, Hamilton, New Zealand.

Rosati, J.D., Dean, R.G., and Walton, T.L., in press. The Modified Bruun Rule Extended for Landward Transport. Marine Geology. doi: 10.1016/j.margeo.2013.04.018

SCOR Working Group 89, 1991. The response of beaches to sea level changes: a review of predictive models. Journal of Coastal Research, 7: 895–921.

Weggel, R., 1979, A method for establishing long-term erosion rates from a long-term rise in water level, CETA: Fort Belvoir, Coastal Engineering Research Center.

Figures

| Figure 1 – Schematic diagram of the components of the Komar et al (1999) Oregon beach erosion model for storm events. |  |
| --- | --- |

| Figure 2 – Schematic diagram of the managed adaptive approach adopted for coastal hazards for the Thames Estuary, United Kingdom. |  |
| --- | --- |